Question 1.

How many networks was each router connected to? Question 1 options: 1 2 3 4

The correct answer is: ✅ 2

Explanation:

Each router connects to two networks:

- The local network (LAN) — where devices like computers and printers are connected.

- The wide area network (WAN) — which connects to another router or the internet.

So every router serves as a bridge between two networks — one on each side.

Question 2.

When sending a message from network C to network A, the Next Hop setup in Router 1 is in: Question 2 options: Network A, Router 0 Network B, Router 0 Network C, Router 1 Network B, Router 1

The correct answer is: ✅ Network B, Router 0

Explanation:

When a message travels from Network C to Network A, it passes through Router 1 → Router 0 → Network A.

Here’s the path:

Network C ── Router 1 ── Network B ── Router 0 ── Network A

- Router 1 receives the packet from Network C.

- To reach Network A, Router 1 must forward it to Router 0 via Network B.

- Therefore, the Next Hop for Router 1 is Router 0 in Network B.

Question 3.

The purpose of an entry in the routing table is to tell the router Question 3 options: where to send the data for a given device address where to send the data for a given MAC address where to send the data for a given traffic type where to send the data for a given network address

The correct answer is: ✅ where to send the data for a given network address

Explanation:

A routing table is used by a router to determine the best path to forward packets based on their destination network address (usually an IP network).

- It does not list individual device or MAC addresses — that’s what an ARP table does.

- It also doesn’t depend on the traffic type (like HTTP, FTP, etc.).

So, in short:

🧭 The routing table tells the router which next hop or interface to use for a given destination network.

Question 4.

The fields type and code, were in which protocol? Question 4 options: IP Ethernet ICMP UDP

The correct answer is: ✅ ICMP

Explanation:

The ICMP (Internet Control Message Protocol) packet includes two key fields:

- Type — specifies the kind of ICMP message (e.g., Echo Request, Echo Reply, Destination Unreachable, etc.)

- Code — provides additional detail about the message type (e.g., the reason for “Destination Unreachable”).

These fields help identify what kind of control or error message is being sent.

💡 Example:

- Type = 8, Code = 0 → Echo Request (ping)

- Type = 0, Code = 0 → Echo Reply

So:

The Type and Code fields belong to the ICMP protocol.

Question 5.

How many networks there are in the packet tracer activity in this lab? Question 5 options: 2 networks, one per router 1 2 networks, one /24 and one /30 3

The correct answer is: ✅ 3

Explanation:

In a typical Packet Tracer lab where two routers are connected, you usually have:

- Network A — connected to Router 0’s LAN

- Network B — the connection between Router 0 and Router 1 (often a /30 network, for point-to-point link)

- Network C — connected to Router 1’s LAN

So in total:

3 networks — one on each side of the routers, plus the one connecting them.

Question 1: If a packet arrived with TTL of 1 at a router, then this packet

A. will be hold until confirmation to increase TTL				B. will be dropped

C. the router will increase TTL by 1 and forward the packet	D. will be forwarded to the next router

Question 2 : How many networks was each router connected to?

A.1			B. 2			C. 3			D. 4

Question 3 : How many networks there are in the packet tracer activity in this lab?

A. 2 networks, one /24 and one /30			B. 1			C. 3			D. 2 networks, one per router

Question 4 : When sending a message from network C to network A, the Next Hop setup in Router 1 is in:

A. Network A, Router 0	B. Network B, Router 0	C. Network C, Router 1	D. Network B, Router 1

Question 5 : What type of routing you have practiced in this lab?

A. Static routing		B. Dynamic routing	C. RIP routing	D. Network mask routing

Question 2 : The fields type and code, were in which protocol?

A. IP			B. Ethernet			C. ICMP			D. UDP

Question 4 : In task 3, what was the purpose of the ping from the client to the server at the first step?

A. to test the TTL max value										B. to test the TTL min value

C. to test the connectivity between the client and the server			D. to build the ARP tables