Question 1 (1 point)

What is the maximum number of hosts in the network: 10.10.0.0 /29

Question 1 options:

|  | 29 |
| --- | --- |
|  | 3 |
|  | 6 |
|  | 8 |
|  | 2 |

Question 2 (1 point)

Link-local addresses are set by:

Question 2 options:

|  | The ISP |
| --- | --- |
|  | The router's DHCP |
|  | The operating system's DHCP |
|  | The device manufacturer |

Question 3 (1 point)

How does a device use the subnet mask?

Question 3 options:

|  | To divide a network into sub-networks |
| --- | --- |
|  | To determine what is the network and host portion of the IPv4 address |
|  | To connect to the default gateway |
|  | To determine the size of the IP address |

Question 4 (1 point)

What is the directed broadcast address of the network: 192.168.0.1 /24

Question 4 options:

|  | 255.255.255.255 /24 |
| --- | --- |
|  | 192.168.0.0 /24 |
|  | 192.168.255.255 /24 |
|  | 192.168.0.255 /24 |

Question 5 (1 point)

Why are we not using classful addressing anymore?

Question 5 options:

|  | Inefficient use of available IPv4 addresses |
| --- | --- |
|  | Class A addresses were insufficient for large organizations |
|  | Class C addresses were insufficient for home users |
|  | Prefix Length notation was not compatible with it |

Question 6 (1 point)

A device has an IP address: 192.168.1.129 /30 .

What is the network address?

Hint:
192 . 168 . 1 . 129   becomes:
1100 0000 . 1010 1000 . 0000 0001 . 1000 0001 in binary

Question 6 options:

|  | 192.168.1.128 /30 |
| --- | --- |
|  | 192.168.1.0 /30 |
|  | 192.168.1.1 /30 |
|  | 192.168.0.0 /30 |

Question 7 (1 point)

When you ping a loopback addresses:

Question 7 options:

|  | The layers are encapsulated and de-encapsulated locally |
| --- | --- |
|  | A message is sent to the default gateway |
|  | The device is checking connectivity to the internet |
|  | The device is checking the speed of the local network |

Question 8 (1 point)

A destination IP address cannot be used for:

options:

|  | Broadcasts |
| --- | --- |
|  | Unicasts |
|  | Multicasts |
|  | NAT |
|  | Can be used for all |

Question 9 (1 point)

What is the prefix length equivalent to a subnet mask of 255.255.255.0 ?

Question 1 options:

|  | 8 |
| --- | --- |
|  | 3 |
|  | 1 |
|  | 24 |
|  | 765 |