Theory Quiz 7 (IPv6, Subnetting)

Question 1:  Uncompress the following IPv6 address: 2001:0:0:aa20::1/64. Include the mask information.

2001:0000:0000:aa20:0000:0000:0000:0001/64

Question 2:  Which compression rule is correct?

A. The longest set of contiguous hextets of zeros can be replaced with a double-colon.

B. All sets of contiguous hextets of zeros can be replaced with a double-colon.

C. One set of contiguous hextets of zeros can be replaced with a double-colon.

D. The first set of contiguous hextets of zeros can be replaced with a double-colon.

Question 3:  Subnet ID 10.5.22.0/26 was created by borrowing two bits. The subnet mask already includes the borrowed bits. What is the network address of the third subnet?

The answer can be written as a simple IP address or an IP address with /26 (no spaces)

10.5.22.128/26

Question 4:  For the subnet ID 192.168.12.0/29, how many hosts addresses are available? The answer must be a number.  6

Question 5:  What is the purpose of the link local address?

A. It is a unique address which is routable.

B. It is only used in a network segment and is routable.

C. It is only used in a network segment and is non routable.

D. This is equivalent to an IPv4 Link Local address.

Question 6: Given the network address 172.16.0.0/16, how many host bits need to be borrowed to create 8 subnets?		3

Question 7: What is the prefix length if the subnet mask is 255.255.254.0 ?  (Hint: 254 = 255 - 1)

The answer is a number.		23

Question 1：For the subnet ID 192.169.64.0/24 what is the first host address?

Question 1 options: 192.169.64.1

A standard global unicast IPv6 address has the following parameters: Global Routing Prefix=2001:acad:0:, an

Interface ID=d5 and a subnet ID=1. Enter the uncompressed IPv6 address including the mask information

(prefix format).

Answer: 2001:acad:0000:0001:0000:0000:0000:00d5/64

Which compression rule is correct? Question 1 options: The longest set of contiguous hextets of zeros can be replaced with a double-colon. All sets of contiguous hextets of zeros can be replaced with a double-colon. One set of contiguous hextets of zeros can be replaced with a double-colon. The first set of contiguous hextets of zeros can be replaced with a double-colon.

ChatGPT 说：

✅ Correct answer:
One set of contiguous hextets of zeros can be replaced with a double-colon.

Explanation:

In IPv6 address compression, the rule is:

- You can replace only one continuous sequence of one or more all-zero hextets with ::.

- If there are multiple zero sequences, you can only compress one of them—usually the longest—to avoid ambiguity when decompressing.

Example:

Original: 2001:0db8:0000:0000:0000:ff00:0042:8329

Compressed: 2001:db8::ff00:42:8329

If there were another zero sequence later, it cannot also be replaced with another ::.

您说：

A standard global unicast IPv6 address has the following parameters: Global Routing Prefix=2001:acad:0:, an Interface ID=d5 and a subnet ID=1. Enter the uncompressed IPv6 address including the mask information (prefix format).

ChatGPT 说：

已思考 9s

Answer:
2001:acad:0000:0001:0000:0000:0000:00d5/64

您说：

For the subnet ID 192.169.64.0/24 what is the first host address?

ChatGPT 说：

Let’s calculate carefully step by step:

- Network address: 192.169.64.0

- Subnet mask: /24 → 255.255.255.0

- In a /24 network, the first address (192.169.64.0) is the network address — not assignable.

- The first host address is one more than the network address.

So:
192.169.64.0 + 1 = 192.169.64.1

✅ Answer: 192.169.64.1

您说：

What is the prefix length if the subnet mask is 255.255.254.0 ? (Hint: 254 = 255 - 1) The answer is a number.

ChatGPT 说：

Let’s convert the subnet mask 255.255.254.0 to prefix length step by step:

- 255 = 8 bits set to 1

- 255 = 8 bits set to 1

- 254 = 11111110 → 7 bits set to 1

- 0 = 00000000 → 0 bits set to 1

Now sum them:
8 + 8 + 7 + 0 = 23

✅ Answer: 23

您说：

What is the purpose of the link local address? Question 5 options: It is only used in a network segment and is routable. This is equivalent to an IPv4 Link Local address. It is a unique address which is routable. It is only used in a network segment and is non routable.

ChatGPT 说：

✅ Correct answer:
It is only used in a network segment and is non routable.

Explanation:

An IPv6 link-local address (starting with FE80::/10) is:

- Automatically assigned to every IPv6-enabled interface.

- Used only for communication within the same local network segment (link).

- Not routable beyond that link — routers do not forward link-local traffic.

您说：

Subnet ID 10.5.22.0/26 was created by borrowing two bits. The subnet mask already includes the borrowed bits. What is the network address of the third subnet?

ChatGPT 说：

Let's solve this step by step.

Given:

- Original network: 10.5.22.0

- Subnet mask: /26 → 255.255.255.192

- Borrowed 2 bits to create subnets (already included in /26)

Step 1: Determine the number of subnets

- Borrowed 2 bits → 22=4subnets

- Each /26 subnet has a block size of 256-192=64addresses

Step 2: List the subnets

- Subnet 1: 10.5.22.0 → range 10.5.22.0 – 10.5.22.63

- Subnet 2: 10.5.22.64 → range 10.5.22.64 – 10.5.22.127

- Subnet 3: 10.5.22.128 → range 10.5.22.128 – 10.5.22.191

- Subnet 4: 10.5.22.192 → range 10.5.22.192 – 10.5.22.255

✅ Step 3: Answer

The network address of the third subnet is:
10.5.22.128

您说：

Subnet ID 10.5.22.0/26 was created by borrowing two bits. The subnet mask already includes the borrowed bits. What is the network address of the third subnet? The answer can be written as a simple IP address or an IP address with /26 (no spaces)

ChatGPT 说：

Step by step:

- Original network: 10.5.22.0

- Subnet mask: /26 → 255.255.255.192

- Block size: 256 − 192 = 64

Subnets:

- 10.5.22.0/26

- 10.5.22.64/26

- 10.5.22.128/26 ← third subnet

- 10.5.22.192/26

✅ Answer:
10.5.22.128/26

您说：

Given the network address 172.16.0.0/16, how many host bits need to be borrowed to create 8 subnets?

ChatGPT 说：

Let's solve this step by step:

Given:

- Network: 172.16.0.0/16

- Goal: 8 subnets

Step 1: Determine how many bits are needed to create 8 subnets

- Formula: 2n≥number of subnets

- 2n≥8→ n=3

Step 2: Conclusion

- 3 bits need to be borrowed from the host portion to create 8 subnets.

✅ Answer: 3

Which compression rule is correct? Question 1 options: The longest set of contiguous hextets of zeros can be replaced with a double-colon. All sets of contiguous hextets of zeros can be replaced with a double-colon. One set of contiguous hextets of zeros can be replaced with a double-colon. The first set of contiguous hextets of zeros can be replaced with a double-colon.

ChatGPT 说：

✅ Correct answer:
One set of contiguous hextets of zeros can be replaced with a double-colon.

Explanation:

In IPv6 address compression, the rule is:

- You can replace only one continuous sequence of one or more all-zero hextets with ::.

- If there are multiple zero sequences, you can only compress one of them—usually the longest—to avoid ambiguity when decompressing.

Example:

Original: 2001:0db8:0000:0000:0000:ff00:0042:8329

Compressed: 2001:db8::ff00:42:8329

If there were another zero sequence later, it cannot also be replaced with another ::.

您说：

A standard global unicast IPv6 address has the following parameters: Global Routing Prefix=2001:acad:0:, an Interface ID=d5 and a subnet ID=1. Enter the uncompressed IPv6 address including the mask information (prefix format).

ChatGPT 说：

已思考 9s

Answer:
2001:acad:0000:0001:0000:0000:0000:00d5/64

您说：

For the subnet ID 192.169.64.0/24 what is the first host address?

ChatGPT 说：

Let’s calculate carefully step by step:

- Network address: 192.169.64.0

- Subnet mask: /24 → 255.255.255.0

- In a /24 network, the first address (192.169.64.0) is the network address — not assignable.

- The first host address is one more than the network address.

So:
192.169.64.0 + 1 = 192.169.64.1

✅ Answer: 192.169.64.1

您说：

What is the prefix length if the subnet mask is 255.255.254.0 ? (Hint: 254 = 255 - 1) The answer is a number.

ChatGPT 说：

Let’s convert the subnet mask 255.255.254.0 to prefix length step by step:

- 255 = 8 bits set to 1

- 255 = 8 bits set to 1

- 254 = 11111110 → 7 bits set to 1

- 0 = 00000000 → 0 bits set to 1

Now sum them:
8 + 8 + 7 + 0 = 23

✅ Answer: 23

您说：

What is the purpose of the link local address? Question 5 options: It is only used in a network segment and is routable. This is equivalent to an IPv4 Link Local address. It is a unique address which is routable. It is only used in a network segment and is non routable.

ChatGPT 说：

✅ Correct answer:
It is only used in a network segment and is non routable.

Explanation:

An IPv6 link-local address (starting with FE80::/10) is:

- Automatically assigned to every IPv6-enabled interface.

- Used only for communication within the same local network segment (link).

- Not routable beyond that link — routers do not forward link-local traffic.

您说：

Subnet ID 10.5.22.0/26 was created by borrowing two bits. The subnet mask already includes the borrowed bits. What is the network address of the third subnet?

ChatGPT 说：

Let's solve this step by step.

Given:

- Original network: 10.5.22.0

- Subnet mask: /26 → 255.255.255.192

- Borrowed 2 bits to create subnets (already included in /26)

Step 1: Determine the number of subnets

- Borrowed 2 bits → 22=4subnets

- Each /26 subnet has a block size of 256-192=64addresses

Step 2: List the subnets

- Subnet 1: 10.5.22.0 → range 10.5.22.0 – 10.5.22.63

- Subnet 2: 10.5.22.64 → range 10.5.22.64 – 10.5.22.127

- Subnet 3: 10.5.22.128 → range 10.5.22.128 – 10.5.22.191

- Subnet 4: 10.5.22.192 → range 10.5.22.192 – 10.5.22.255

✅ Step 3: Answer

The network address of the third subnet is:
10.5.22.128

您说：

Subnet ID 10.5.22.0/26 was created by borrowing two bits. The subnet mask already includes the borrowed bits. What is the network address of the third subnet? The answer can be written as a simple IP address or an IP address with /26 (no spaces)

ChatGPT 说：

Step by step:

- Original network: 10.5.22.0

- Subnet mask: /26 → 255.255.255.192

- Block size: 256 − 192 = 64

Subnets:

- 10.5.22.0/26

- 10.5.22.64/26

- 10.5.22.128/26 ← third subnet

- 10.5.22.192/26

✅ Answer:
10.5.22.128/26

您说：

Given the network address 172.16.0.0/16, how many host bits need to be borrowed to create 8 subnets?

ChatGPT 说：

Let's solve this step by step:

Given:

- Network: 172.16.0.0/16

- Goal: 8 subnets

Step 1: Determine how many bits are needed to create 8 subnets

- Formula: 2n≥number of subnets

- 2n≥8→ n=3

Step 2: Conclusion

- 3 bits need to be borrowed from the host portion to create 8 subnets.

✅ Answer: 3